Homework 4

Ashmitha Murali

Prof. Alex Yang

Digital Design and HDL EE461

5th November 2025

Q1.

Result:

Code:

RTL level module:

`timescale 1ns/1ps

module detect_all_rtl (

input  wire [7:0] din,

output wire       zeroflag,

output wire       oneflag

);

assign zeroflag = ~|din;

assign oneflag  =  &din;

endmodule

Gate Level module:

module detect_all_gates (

input  wire [7:0] din,

output wire       zeroflag,

output wire       oneflag

);

wire or01, or23, or45, or67;

wire or0123, or4567, or_all;

or  (or01,   din[0], din[1]);

or  (or23,   din[2], din[3]);

or  (or45,   din[4], din[5]);

or  (or67,   din[6], din[7]);

or  (or0123, or01,   or23);

or  (or4567, or45,   or67);

or  (or_all, or0123, or4567);

not (zeroflag, or_all);

wire and01, and23, and45, and67;

wire and0123, and4567;

and (and01,   din[0], din[1]);

and (and23,   din[2], din[3]);

and (and45,   din[4], din[5]);

and (and67,   din[6], din[7]);

and (and0123, and01,  and23);

and (and4567, and45,  and67);

and (oneflag, and0123, and4567);

endmodule

Testbench:

module tb_detect_all_0_1_simple;

reg  [7:0] din;

wire zeroflag_rtl, oneflag_rtl;

wire zeroflag_gate, oneflag_gate;

detect_all_rtl   dut_rtl  (.din(din), .zeroflag(zeroflag_rtl), .oneflag(oneflag_rtl));

detect_all_gates dut_gate (.din(din), .zeroflag(zeroflag_gate), .oneflag(oneflag_gate));

initial begin

$display("Time | Input | RTL (zeroflag, oneflag) | Gate (zeroflag, oneflag)");

$display("-------------------------------------------------");

din = 8'b00000000;

#1;

$display("%4t | %b | (%0d,%0d) | (%0d,%0d)",

$time, din, zeroflag_rtl, oneflag_rtl, zeroflag_gate, oneflag_gate);

din = 8'b11111111;

#1;

$display("%4t | %b | (%0d,%0d) | (%0d,%0d)",

$time, din, zeroflag_rtl, oneflag_rtl, zeroflag_gate, oneflag_gate);

din = 8'b10101010;

#1;

$display("%4t | %b | (%0d,%0d) | (%0d,%0d)",

$time, din, zeroflag_rtl, oneflag_rtl, zeroflag_gate, oneflag_gate);

$finish;

end

endmodule

Q2.

Result:

Code:

module twos_complement_4bit (

input  wire [3:0] A,

output wire [3:0] Y

);

//Bitwise inversion

wire [3:0] inv;

not (inv[0], A[0]);

not (inv[1], A[1]);

not (inv[2], A[2]);

not (inv[3], A[3]);

//full adders to add 1

wire c0, c1, c2;

xor (Y[0], inv[0], 1'b1);

and (c0, inv[0], 1'b1);

xor (Y[1], inv[1], c0);

and (c1, inv[1], c0);

xor (Y[2], inv[2], c1);

and (c2, inv[2], c1);

xor (Y[3], inv[3], c2);

endmodule

Testbench:

module tb_twos_complement_4bit;

reg  [3:0] A;

wire [3:0] Y;

twos_complement_4bit dut (.A(A), .Y(Y));

initial begin

$display("Time | Input(A) | Two's Complement (Y)");

$display("--------------------------------------");

A = 4'b0000; #1;  // 0 → 0

$display("%4t | %b | %b", $time, A, Y);

A = 4'b0001; #1;  // +1 → -1 (1111)

$display("%4t | %b | %b", $time, A, Y);

A = 4'b0010; #1;  // +2 → -2 (1110)

$display("%4t | %b | %b", $time, A, Y);

A = 4'b0101; #1;  // +5 → -5 (1011)

$display("%4t | %b | %b", $time, A, Y);

A = 4'b1110; #1;  // -2 → +2 (0010)

$display("%4t | %b | %b", $time, A, Y);

A = 4'b1000; #1;  // -8 → +8

$display("%4t | %b | %b", $time, A, Y);

$finish;

end

endmodule

Q3.

Result:

When we assign x or z to select lines the output will be assigned to x. This is because when a select line bits is neither 1 or 0 then the output will be unknown.

Code:

module mux4to1 (

input  wire [3:0] D,

input  wire [1:0] S,

output wire       Y

);

assign Y = (S == 2'b00) ? D[0] :

(S == 2'b01) ? D[1] :

(S == 2'b10) ? D[2] :

(S == 2'b11) ? D[3] : 1'bx;

endmodule

Testbench:

module tb_mux4to1;

reg  [3:0] D;

reg  [1:0] S;

wire Y;

mux4to1 uut (.D(D), .S(S), .Y(Y));

initial begin

$display("Time | S | D | Y");

$display("------------------");

D = 4'b1010;

S = 2'b00; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

S = 2'b01; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

S = 2'b10; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

S = 2'b11; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

S = 2'bx0; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

S = 2'b0x; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

S = 2'bzz; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

S = 2'bxz; #1; $display("%4t | %b | %b | %b", $time, S, D, Y);

$finish;

end

endmodule

Q4.

- always @(a, b, c) begin
d = a + b;
e = d + c;
end

- b. always @(a, b, c) begin
e = d + c;
d = a + b;
end

Since we are using blocking assignments the order of the assignments matter.

In the first question a+b is assigned to d and the updated value of d is added with c to get e.

But in the second question d and c values are added first and then d is updated with a+b. Hence we need to use old value of d to get e. Hence we implement a latch that stores previous value of d to be added to c.

Q5.

Since we have case statement where all the input combinations for 3 bit select lines are not in the case, a latch will be formed at the output. To prevent a latch from forming, we can tie down the output to 0 when one of the cases is not specified.

module mux(

input  wire a,b,c,d,e,

input  wire [2:0] sel,

output reg  out

);

always @* begin

// Option A: give a default first (no latch even if case misses)

out = 1'b0;          // choose a defined default for 101/110/111

case (sel)

3'b000: out = a;

3'b001: out = b;

3'b010: out = c;

3'b011: out = d;

3'b100: out = e;

// no need for default here since we preassigned out

endcase

end

endmodule

Schematic: